Duet Module Interface Specification

for a

Marantz PD4250D

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	3

Channels	5

Levels	6

Command Control	7

Command Feedback	10

Device Notes	11

Programming Notes	11

Protocol Notes	11

Adding Functions to Modules	11

Commands to the device	11

LIST OF TABLES

Table 1 - Virtual Device Channel Events	5

Table 2 - Virtual Device Level Events	6

Table 3 – Send Command Definitions	9

Table 4 - Command Feedback Definitions	10

Revision History

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 12-01-2005 | YS | Initial release |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a Marantz PD4250D plasma monitor. The Marantz PD4250D plasma monitor supports an RS-232 serial protocol. The required communication settings are a baud rate of 9600, 8 data bits, 1 stop bit, no parity, and handshaking off. The cable for this device is FG#756-04.

## Overview

The COMM module translates between the standard interface described below and the Marantz PD4250D plasma monitor serial protocol. It parses the buffer for responses from the monitor, sends strings to control the monitor, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Duet module.

Marantz PD4250D plasma

Duet

COMM Module

NetLinx

UI Module

Virtual

Device

SNAPI

Some functionality in the device interface may not be implemented in the API interface.  In cases where device functions are desired but not API-supported, the PASSTHRU command may be used to send any and all device-protocol commands to the device. See the PASSTHRU command and the Adding Functions to Modules section for more information.

A sample UI module and a touch panel file are provided in the module package.  These are not intended to cover every possible application, but can be expanded as needed by a dealer to meet the requirements of a particular installation.

## Implementation

To interface to the AMX Marantz PD4250D plasma monitor module, the programmer must perform the following steps:

- Define the device ID for the Marantz PD4250D plasma monitor that will be controlled.

- Define the virtual device ID that the Marantz PD4250D plasma monitor COMM module will use to communicate with the main program and User Interface. Duet virtual devices use device numbers 41000 - 42000.

- If a touch panel interface is desired, a touch panel file Marantz_PD4250D.TP4 and module (Marantz_PD4250D.axs) have been created for testing.

- The Duet Marantz_PD4250D module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the monitor to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

dvTouchPanel 	= 10001:1:0	// The touch panel used for output

dvDevice		= 5001:1:0	// The switcher connected to the NetLinx on 1st RS-232 port

vdvDevice       	= 41001:1:0	// The virtual device use for communication between the

// Comm module interface and User_Interface (UI) module interface

DEFINE_VARIABLE

//Define arrays of button channels used on your own touch panel

integer nTP_BUTTONS[]={1,2,3,4,5,6,7}

DEFINE_START	// Place define_module calls to the very end of the define_start section.

// Comm module

DEFINE_MODULE 'Marantz_PD4250D_Comm_v1_0_0' DuetCode(vdvDevice, dvDevice)

DEFINE_MODULE 'Marantz_PD4250D_UI'   TP1(vdvDevice, dvTP, nCHAN_BTN, nTXT_BTN, nLVL_BTN)

Upon initialization the AMX Comm module will communicate with the plasma monitor and information will be exchanged.

## Channels

The UI module controls the Marantz PD4250D via channel events (NetLinx commands pulse, on, and off) sent to the COMM module. The channels supported by the COMM module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device.

| Channel | Description |
| --- | --- |
| 9 | PULSE: Cycle Power |
| 26 | PULSE: Cycle Volume Mute |
| 27 | ON: Set Power On |
| 28 | ON: Set Power Off |
| 31 | PULSE: Set Input Source to Video, 1 |
| 32 | PULSE: Set Input Source to Video, 2 |
| 33 | PULSE: Set Input Source to Video, 3 |
| 142 | PULSE: Cycle Aspect Ratio |
| 148 | PULSE: Increment Brightness |
| 149 | PULSE: Decrement Brightness |
| 150 | PULSE: Increment Color |
| 151 | PULSE: Decrement Color |
| 152 | PULSE: Increment Contrast |
| 153 | PULSE: Decrement Contrast |
| 154 | PULSE: Increment Sharpness |
| 155 | PULSE: Decrement Sharpness |
| 156 | PULSE: Increment Tint |
| 157 | PULSE: Increment Tint |
| 196 | PULSE: Cycle Input Source |
| 199 | ON: Set Volume Mute On – used for feedback also OFF: Set Volume Mute Off |
| 251 | ON: Device is Online – provides feedback only OFF: Device is not Online |
| 252 | ON: Data is Initialized – provides feedback only OFF: Data is not Initialized |
| 255 | ON: Set power on – used for feedback also OFF: Set power off |

Table 1 - Virtual Device Channel Events

^ - This device does not natively support volume ramping. Therefore, the ramping channels will force the device to do an increment or decrement adjustment to the volume and then force the ramping channel off.

## Levels

The UI module controls the Marantz PD4250D via level events (NetLinx command send_level) sent to the COMM module. The levels supported by the COMM module are listed below. These levels are associated with the virtual device(s) and are independent of the levels associated with the touch panel device.

| Level | Description |
| --- | --- |
| 1 | Volume level – 0..255 |
| 10 | Brightness level – 0..255 |
| 11 | Color level – 0..255 |
| 12 | Contrast level – 0..255 |
| 13 | Sharpness level – 0..255 |
| 14 | Tint level – 0..255 |

Table 2 - Virtual Device Level Events

## Command Control

The UI module controls the monitor via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

| Command | Description |
| --- | --- |
| ?ASPECT | Get the current aspect ratio. Note: Make sure you have an active input and an aspect ratio has been selected first. Note: The device does not give initial feedback on the current aspect ratio so until the aspect ratio is changed once after master reboot the aspect ratio will be invalid. ?ASPECT |
| ASPECT-<ratio> | Set the current aspect ratio. Note: Make sure you have an active input and an aspect ratio has been selected first. <ratio> :STADIUM ZOOM NORMAL ANAMORPHIC CUSTOM – 14:9 aspect ratio ANAMORPHIC2 – 2.35:1 aspect ratio ASPECT-NORMAL |
| ?DEBUG | Request the state of the debug feature. ?DEBUG |
| DEBUG-<value> | Set the state of debugging messages in the UI module and the Comm. module. Note: See Programming Notes section. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning & info messages on 4 = set all messages on DEBUG-1 |
| ?INPUT | Get the currently selected input source. ?INPUT |
| INPUT-<source>,<number> | Set the current input source. <source> : VIDEO HDTV COMPUTER <number> : 1 – 3 inputs 1 thru 3 for VIDEO and HDTV, only input 1 for COMPUTER INPUT-VIDEO,2 |
| PASSBACK-<state> | Enable or disable response reporting from the device. When enabled device responses will be sent as strings to the virtual device. Note: By default, this is set to off at startup. <state> : 0 = Off (default) 1 = On PASSBACK-0 |
| PASSTHRU-<string> | Allows user the capability of sending commands directly to whatever unit is attached without processing by the Duet module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features that may not be directly supported by the module. For more information, see the “Adding Functions to Modules” section below. NOTE: The module will calculate the checksum for you. <string> : string to send to unit "'passthru-',$Df,$80,$60,$47,$01,$05" – input HD1 |
| PROPERTY-<key>,<value> | Set the value of a particular property stored in the module. Note: <key> is case sensitive. Note: The default Polling rate is every 5 seconds. <key> : POLLING = module’s polling rate. <value> : 0 = disable polling 10-3600 = polling time in seconds PROPERTY-POLLING,2 |
| ?PROPERTY-<key> | Query for the current value stored in a particular property. Note: The default Polling rate is every 5 seconds. <key> : POLLING = module’s polling rate. ?PROPERTY-POLLING |
| REINIT | Re-initializes the communication link and data. Note: This command deletes any messages waiting to go out to the device. REINIT |
| ?VERSION | Query for the current version number of the Duet module. ?VERSION |

Table 3 – Send Command Definitions

## Command Feedback

The COMM module provides feedback to the User Interface module for the monitor changes via command events. The commands supported are listed below.

PLEASE NOTE: Feedback is only provided when there is a state change. If no state change resulted from the command sent in, then no feedback will be returned.

| Command | Description |
| --- | --- |
| ASPECT-<ratio> | Reports the current aspect ratio. <ratio> :STADIUM ZOOM NORMAL ANAMORPHIC CUSTOM – 14:9 aspect ratio ANAMORPHIC2 – 2.35:1 aspect ratio ASPECT-NORMAL |
| DEBUG-<value> | Returns the state of debugging messages in the UI module and the Comm. module. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning and info messages on 4 = set all messages on DEBUG-1 |
| INPUT-<source>,<number> | Returns the current input source. <source> : VIDEO HDTV COMPUTER <number> : 1 – 3 inputs 1 thru 3 for VIDEO and HDTV, only input 1 for COMPUTER INPUT-VIDEO,2 |
| PROPERTY-<key>,<value> | Feedback on the value of a particular property stored in the module. Note: The default Polling rate is every 5 seconds. <key> : POLLING = module’s polling rate. <value> : 0 = polling is disabled 10-3600 = polling time in seconds PROPERTY-POLLING,2 |
| VERSION-<version> | Reports the version number of the module. <version> : xx.yy.zz = module version number VERSION-1.0.0 |

Table 4 - Command Feedback Definitions

## Device Notes

	There is no way to query for the current volume of the plasma monitor that is why the volume up/down is not supported, you can only set the volume directly using the active level bar, any changes from the TV/Remote to the volume level will not get updated on the TP.

	There is no way to query for the status of mute. That is why mute is a momentary button.

	There is no way to query for the aspect ratio, the only feedback for aspect ratio is when you select an aspect ratio from the TP, that is why the button gets updated, so any changes to the aspect ratio from the TV/Remote will not be reflected on the TP, nor on boot up of the master.

## Programming Notes

	The module will send a full query after every time the master gets rebooted, the full query includes: Power status, current input, video status which includes (contrast level, brightness level, color level, tint level, sharpness level).

	For Sharpness, it has two different ranges; the range is automatically selected depending on the current input (Video vs. RGB).

## Protocol Notes

None

## Adding Functions to Modules

### Commands to the device

This module supplies a mechanism to allow additional device features to be added to software using the module.  This is the ‘PASSTHRU-‘ command, which allows protocol strings to be passed through the module.  The device-specific protocol must be known in order to use this feature.

As an example, suppose that a module for a projector has not implemented the 'white balance adjustment' feature.   The command that the projector protocol requires is 03H, 10H, 05H, 14H, followed by a checksum.   The documentation for the ‘PASSTHRU-‘ command specifies that the module will automatically generate the checksum.   In this case, the following string should be sent from the UI code to implement 'white balance adjustment'.

send_command vdvDevice, "'PASSTHRU-',$03,$10,$05,$14"

The reason to use ‘PASSTHRU-‘ instead of sending a protocol string directly to the device port is that the device may require command queuing, calculation of checksums, or other internal processing, which would not be done if the string was sent directly. Because of this, it is best to filter all communication TO the device through the module. (The module documentation will indicate any processing that will be automatically done to the ‘PASSTHRU-‘ command like checksum calculation.)