# DENON SYSTEM control protocol

Ver.1.1.0

Application model :　FY17 Model（ND8006）

Application terminal: RS-232C/ Ethernet

## Connector specification

Ⅰ. RS-232C

Connector type:  DB-9pin female type, slave straight connection (DCE type)

( 1pin : GND , 2pin : TxD , 3pin : RxD , 5pin : Common(GND) , 4,6,7,8,9pin : NC )

Communication format:

Synchronous system			: Tone step synchronization

Communication system		: A half duplex

Communication speed 		: 9600bps

Character length 			: 8 bits

Parity control 			: None

Start bit 			: 1 bit

Stop bit 				: 1 bit

Communication procedure 		: Non procedural

Communication data length 		: 135 bytes (maximum)

Ⅱ. Ethernet

Connector type   ：  RJ-45(10BASE-T/100BASE-TX)

Communication format  ：

Communication system 		: A half duplex

Communication speed 		: 10Mbps/100Mbps

Communication port	         : TCP port 23 (telnet)

Communication data length      : 135bytes (maximum)

| Version | Date | Contents | Change Point |
| --- | --- | --- | --- |
| 1.0.1 | 2015/1/5 | Original | ALL |
| 1.0.2 | 2015/1/5 | Added FY15 model | ALL |
| 1.0.3 | 2016/12/7 | Modify for ND8005 | ALL |
| 1.0.4 | 2016/12/14 | Modify based on feedback from Y.Komatsu | MV,SI |
| 1.0.5 | 2016/12/14 | Remove discrive of Network Setup | <- |
| 1.0.6 | 2016/12/15 | Modify based on feedback from Y.Komatsu | MV,SI |
| 1.0.7 | 2016/12/17 | Modify based on feedback from Y.Komatsu | MV (Event部) |
| 1.0.8 | 2017/ 1/26 | Added some commands | MV, MVLONG , IRTX, RCRC5, SSFMTALC |
| 1.0.9 | 2017/ 2/2 | Modified some commands | SI, MVLONG, SSSTB, IRTX, RCRC5, SSFMT |

Protocol specification

The following three data forms are defined.

COMMAND 　: 	The message sent to a system from a controller(Touch Panel etc.)

A command to a system is given from a controller.

EVENT     :	The message sent to a controller (Touch Panel etc.) from a system

The result is sent, when a system is operated directly and a state changes.

*The form of EVENT presupposes that it is the same as that of COMMAND.

**Refer to the following table for the contents of COMMAND and EVENT.

RESPONSE :	The message sent to a controller (Touch Panel etc.) from a system

if the ‘request command’ (COMMAND+?+CR(0x0D)) has came from a controller.

The RESPONSE should be sent within 200ms of receiving the COMMAND.

*The form of RESPONSE presupposes that it is the same as that of EVENT.

Basic specification: The command by ASCII CODE, parameter expression

*ASCII CODE which can be used is from 0x20 to 0x7F: the alphabet and the number of 0-9, and space (0x20), some signs,

AND carriage return (0x0D) --- It is used only as a pause sign.

Command structure: COMMAND + PARAMETER + CR (0x0D)

COMMAND: ASCII CODE of 2 characters

Ex. 	SI  : 	Select Input source

PW  :	system PoWer setting

PARAMETER : ASCII CODE ( up to 25 characters)

ex.	IRADIO: function name

*Special Parameter--- ? : for request command

The example of a command		* <CR> is the meaning of 0x0D.

PWON<CR>       :  system Power ON

PWSTANDBY<CR>  :  system Power STANDBY

SI?<CR>        :  Request command for now playing input source >> Return RESPONSE ‘SI***<CR>’

Others

- COMMAND is receivable also during transmission of EVENT.

- The RESPONSE should be sent as opposed to the request command by all the commands with which an EVENT exists , not need to the another request commands(ex. SI command).

# COMMAND and PARAMETER list

## ※SYSTEM関係

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| PW | ON | POWER ON/STANDBY change | PWON<CR> |
|  | STANDBY |  | PWSTANDBY<CR> |
|  | ? | Return PW Status | PW?<CR> |
| MV | UP | MASTER VOLUME UP/DOWN , direct change to **dB | MVUP<CR> |
|  | DOWN |  | MVDOWN<CR> |
|  | ** | **:00 to 60 by ASCII | MV45<CR> |
|  | ? | Return MV Status | MV?<CR> |
| MU | ON | OUTPUT MUTE ON/OFF change | MUON<CR> |
|  | OFF |  | MUOFF<CR> |
|  | ? | Return MU Status | MU?<CR> |
| SI | Alternative command need to be added in CLI. (This part cannot operate via Serial RS232C control. Same as AVR.) | Heos Favorites | [None] |
|  |  | Internet Radio |  |
|  |  | Music Server |  |
|  |  | USB |  |
|  |  | Music Services |  |
|  | BT | Bluetooth | SIBT<CR> |
|  | CD | CD | SICD<CR> |
|  | PC | PC(USB-DAC) | SIPC<CR> |
|  | COAXIAL | Coaxial | SICOAXIAL<CR> |
|  | OPTICAL1 | Optical 1 | SIOPTICAL1<CR> |
|  | OPTICAL2 | Optical 2 | SIOPTICAL2<CR> |
|  | ? | Return SI Status | SI?<CR> |
| SLP | OFF | MAIN ZONE SLEEP TIMER setting | SLPOFF<CR> |
|  | *** | ***:001 to 090 by ASCII , 010=10min | SLP***<CR> |
|  | ? | Return SLP Status | SLP?<CR> |

## ※FAVORITE Control : CLI use (This part cannot operate via Serial RS232C control. Same as AVR.)

## ※Variable Output

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | VVL | Variable Output（Volume Limit）:0dB | SSVVL 000 |
|  |  | Variable Output（Volume Limit）:-10dB | SSVVL 010 |
|  |  | Variable Output（Volume Limit）:-20dB | SSVVL 020 |
|  |  | Volume Limit Direct | SSVVL *** （***：Limit） |
|  |  | ? | SSVVL ? |

## ※Auto Standby

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | STB | Auto Standby：ON | SSSTB ON<CR> |
|  |  | Auto Standby：OFF | SSSTB OFF<CR> |
|  |  | Auto Standby：1min | SSSTB 01<CR> |
|  |  | Auto Standby：5min | SSSTB 05<CR> |
|  |  | Auto Standby：15min | SSSTB 15<CR> |
|  |  | Auto Standby Direct（**：TIME 01～99） | SSSTB **<CR> |
|  |  | ? | SSSTB ? <CR> |

## ※Variable Output Volume

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| MV | UP | MASTER VOLUME UP/DOWN , direct change to **dB | MVUP<CR> |
|  | DOWN |  | MVDOWN<CR> |
|  | ** or *** | 00 to 100 by ASCII (0.5step) | MV100<CR> 100.0 MV995<CR> 99.5 MV99<CR> 99.0 MV985<CR> 98.5 MV98<CR> 98.0 : : : : MV015<CR> 1.5 MV01<CR> 1.0 MV005<CR> 0.5 MV00<CR> 0.0 |
|  | ? | Return MV Status | MV?<CR> |
| MVLONG | UP | VOLUME UP(Long push) | MVLONGUP<CR> |
|  | DOWN | VOLUME DOWN(Long push) | MVLONGDOWN<CR> |
|  | END | VOLUME(Long push END) | MVLONGEND<CR> |

## ※Language設定

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | LAN ENG | 英語 | SSLAN ENG<CR> |
|  | LAN DEU | ドイツ語 | SSLAN DEU<CR> |
|  | LAN FRA | フランス語 | SSLAN FRA<CR> |
|  | LAN ITA | イタリア語 | SSLAN ITA<CR> |
|  | LAN ESP | スペイン語 | SSLAN ESP<CR> |
|  | LAN NER | オランダ語 | SSLAN NER<CR> |
|  | LAN SVE | スウェーデン語 | SSLAN SVE<CR> |
|  | LAN JPN | 日本語 | SSLAN JPN<CR> |
|  | LAN CHI | 中国語 | SSLAN CHI<CR> |
|  | LAN POL | ポーランド語 | SSLAN POL<CR> |
|  | LAN RUS | ロシア語 | SSLAN RUS<CR> |
|  | ? | 要求 | SSLAN ? <CR> |

## ※AUDIO 再生フォーマット

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | FMT? | 再生ファイルの種類取得 | SSFMT?<CR> |

## ※DIMMER設定

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | DIM000 | DIMMER：0％ | SSDIM000<CR> |
|  | DIM025 | DIMMER：25％（5段階用） | SSDIM025<CR> |
|  | DIM040 | DIMMER：40％（4段階用） | SSDIM040<CR> |
|  | DIM050 | DIMMER：50％（5段階用） | SSDIM050<CR> |
|  | DIM070 | DIMMER：70％（4段階用） | SSDIM070<CR> |
|  | DIM075 | DIMMER：75％（5段階用） | SSDIM075<CR> |
|  | DIM100 | DIMMER：100％ | SSDIM100<CR> |
|  | DIM? | 要求 | SSDIM?<CR> |

## ※Network Related : CLI use (This part cannot operate via Serial RS232C control. Same as AVR.)

## ※APPS→RC-5 OUT Command （Marantz）

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| MV | UP | VOLUME UP | MVUP<CR> |
|  | DOWN | VOLUME DOWN | MVDOWN<CR> |
| MVLONG | UP | VOLUME UP(Long push) | MVLONGUP<CR> |
|  | DOWN | VOLUME DOWN(Long push) | MVLONGDOWN<CR> |
|  | END | VOLUME(Long push END) | MVLONGEND<CR> |
| IRTX | 1601201 | AMP:(Power) On | IRTX1601201<CR> |
|  | 1601202 | AMP:Standby | IRTX1601202<CR> |
|  | 20063 | AMP:CD | IRTX20063<CR> |
|  | 17063 | AMP:LINE 1 | IRTX17063<CR> |
|  | 1600010 | AMP:LINE 2 | IRTX1600010<CR> |
|  | 26063 | AMP:RECORDER 1 | IRTX26063<CR> |
|  | 18063 | AMP:RECORDER 2 | IRTX18063<CR> |
|  | 21063 | AMP:PHONO | IRTX21063<CR> |
|  | 1600020 | AMP:BALANCED | IRTX1600020<CR> |
|  | 1706310 | AMP:TUNER | IRTX1706310<CR> |
|  | 1600006 | AMP:AUX | IRTX1600006<CR> |
|  | 1600010 | AMP:DVD | IRTX1600010<CR> |
|  | 1601300 | AMP:Mute on | IRTX1601300<CR> |
|  | 1601301 | AMP:Mute Off | IRTX1601301<CR> |
|  | 1616 | AMP:VOL UP | IRTX1616<CR> |
|  | 1617 | AMP:VOL DOWN | IRTX1617<CR> |
|  | 1613 | AMP:ATT(Mute） | IRTX1613<CR> |
|  | 1634 | AMP:Source Direct | IRTX1634<CR> |
|  | 2001201 | CD:Power On | IRTX2001201<CR> |
|  | 2001202 | CD:Power Off | IRTX2001202<CR> |
|  | 20045 | CD:Eject | IRTX20045<CR> |
|  | 20033 | CD:Previous | IRTX20033<CR> |
|  | 20053 | CD:Play | IRTX20053<CR> |
|  | 20032 | CD:Next | IRTX20032<CR> |
|  | 20048 | CD:Pause | IRTX20048<CR> |
|  | 20054 | CD:Stop | IRTX20054<CR> |
|  | 20029 | CD:Repeat | IRTX20029<CR> |
|  | 20028 | CD:Shuffle | IRTX20028<CR> |

## ※Remote Code（C4 Command）（Marantz：RC-5）

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| RCRC5 | 2501200 | Remote Code:ON/STANDBY | RCRC52501200<CR> |
|  | 2501201 | Remote code:Power On (Extended code) | RCRC52501201<CR> |
|  | 2501202 | Remote code:Power Off (Extended code) | RCRC52501202<CR> |
|  | 2503801 | Remote Code:SLEEP | RCRC52503801<CR> |
|  | 2507100 | Remote Code:DIMMER | RCRC52507100<CR> |
|  | 2507101 | Remote code:Dimmer Off (Extended code) | RCRC52507101<CR> |
|  | 2507102 | Remote code:Dimmer Mode 1 (Extended code) | RCRC52507102<CR> |
|  | 2507103 | Remote code:Dimmer Mode 2 (Extended code) | RCRC52507103<CR> |
|  | 2507104 | Remote code:Dimmer Mode 3 (Extended code) | RCRC52507104<CR> |
|  | 2506351 | Remote Code:INTERNET RADIO | RCRC52506351<CR> |
|  | 2506350 | Remote Code:MUSIC SERVER | RCRC52506350<CR> |
|  | 2506321 | Remote Code:USB | RCRC52506321<CR> |
|  | 2506331 | Remote Code: OPTICAL1 | RCRC52506331<CR> |
|  | 2506333 | Remote Code: OPTICAL2 | RCRC52506333<CR> |
|  | 2506332 | Remote Code: COAXIAL | RCRC52506332<CR> |
|  | 2506330 | Remote code:OPT/COAX (Extended code) | RCRC52506330<CR> |
|  | 2506320 | Remote Code:CD | RCRC52506320<CR> |
|  | 2506338 | Remote Code:Bluetooth | RCRC52506338<CR> |
|  | 2506337 | Remote Code:PC(USB-DAC) | RCRC52506337<CR> |
|  | 2506300 | Remote code:Input select up (Extended code) | RCRC52506300<CR> |
|  | 2506301 | Remote code:Input select down (Extended code) | RCRC52506301<CR> |
|  | 2503300 | Remote Code:Skip- | RCRC52503300<CR> |
|  | 2505300 | Remote Code:Play/Pause | RCRC52505300<CR> |
|  | 2505301 | Remote code:Play (Extended code) | RCRC52505301<CR> |
|  | 2504801 | Remote code:Pause (Extended code) | RCRC52504801<CR> |
|  | 2503200 | Remote Code:Skip+ | RCRC52503200<CR> |
|  | 2505000 | Remote code:FR (Extended code) | RCRC52505000<CR> |
|  | 2505400 | Remote Code:Stop | RCRC52505400<CR> |
|  | 2505200 | Remote code:FF (Extended code) | RCRC52505200<CR> |
|  | 2511922 | Remote Code:FAVORITES ADD | RCRC52511922<CR> |
|  | 2511921 | Remote Code:FAVORITES CALL | RCRC52511921<CR> |
|  | 2511920 | Extended code:Favorite (Extended code) | RCRC52511920<CR> |
|  | 2501310 | Remote Code:VOLUME MUTE | RCRC52501310<CR> |
|  | 2501610 | Remote Code:VOLUME UP | RCRC52501610<CR> |
|  | 2501710 | Remote Code:VOLUME DOWN | RCRC52501710<CR> |
|  | 2503610 | Remote Code:QUEUE | RCRC52503610<CR> |
|  | 2508710 | Remote Code:BACK | RCRC52508710<CR> |
|  | 2508220 | Remote Code:OPTION | RCRC52508220<CR> |
|  | 2508200 | Remote Code:SETUP | RCRC52508200<CR> |
|  | 2508000 | Remote Code:cursor UP | RCRC52508000<CR> |
|  | 2508100 | Remote Code:cursor DOWN | RCRC52508100<CR> |
|  | 2508500 | Remote Code:cursor LEFT | RCRC52508500<CR> |
|  | 2508600 | Remote Code:cursor RIGHT | RCRC52508600<CR> |
|  | 2508700 | Remote Code:ENTER | RCRC52508700<CR> |
|  | 2500100 | Remote Code:1 | RCRC52500100<CR> |
|  | 2500200 | Remote Code:2 ABC | RCRC52500200<CR> |
|  | 2500300 | Remote Code:3 DEF | RCRC52500300<CR> |
|  | 2500400 | Remote Code:4 GHI | RCRC52500400<CR> |
|  | 2500500 | Remote Code:5 JKL | RCRC52500500<CR> |
|  | 2500600 | Remote Code:6 MNO | RCRC52500600<CR> |
|  | 2500700 | Remote Code:7 PQRS | RCRC52500700<CR> |
|  | 2500800 | Remote Code:8 TUV | RCRC52500800<CR> |
|  | 2500900 | Remote Code:9 WXYZ | RCRC52500900<CR> |
|  | 2501000 | Remote Code:+10 a/A | RCRC52501000<CR> |
|  | 2500000 | Remote Code:0 | RCRC52500000<CR> |
|  | 2504900 | Remote Code:CLEAR | RCRC52504900<CR> |
|  | 2503600 | Remote Code:PROGRAM | RCRC52503600<CR> |
|  | 2503601 | Remote Code:Program On (Extended code) | RCRC52503601<CR> |
|  | 2503602 | Remote Code:Program Off (Extended code) | RCRC52503602<CR> |
|  | 2502800 | Remote Code:RANDOM | RCRC52502800<CR> |
|  | 2502801 | Remote Code:Random On (Extended code) | RCRC52502801<CR> |
|  | 2502802 | Remote Code:Random Off (Extended code) | RCRC52502802<CR> |
|  | 2502900 | Remote Code:REPEAT | RCRC52502900<CR> |
|  | 2502902 | Remote code:Repeat 1 (Extended code) | RCRC52502902<CR> |
|  | 2502903 | Remote code:Repeat All (Extended code) | RCRC52502903<CR> |
|  | 2502901 | Remote code:Repeat Off (Extended code) | RCRC52502901<CR> |
|  | 2501100 | Remote Code:INFO | RCRC52501100<CR> |
|  | 2507700 | Remote code:Digital Out (Toggle) (Extended code) | RCRC52507700<CR> |
|  | 2507701 | Remote code:Digital Out On (Extended code) | RCRC52507701<CR> |
|  | 2507702 | Remote code:Digital Out Off (Extended code) | RCRC52507702<CR> |
|  | 2511930 | Remote Code:MODE (Extended code) | RCRC52511930<CR> |
|  | 2506352 | Remote Code:ONLINE MUSIC | RCRC52506352<CR> |
|  | 2508720 | Remote Code:TOP MENU | RCRC52508720<CR> |
|  | 2503210 | Remote Code:SEARCH | RCRC52503210<CR> |
|  | 2506355 | Extended code:(input) Pandora | RCRC52506355<CR> |
|  | 2506356 | Extended code:(input) Last.fm | RCRC52506356<CR> |
|  | 2506357 | Extended code:(input) SiriusXM | RCRC52506357<CR> |
|  | 2506358 | Extended code:(input) Spotify | RCRC52506358<CR> |
|  | 2508210 | Extended code:iPod mode (Toggle) | RCRC52508210<CR> |
|  | 2508211 | Extended code:iPod Direct mode | RCRC52508211<CR> |
|  | 2508212 | Extended code:iPod Remote mode | RCRC52508212<CR> |
|  | 16012 | Remote Code:AMP (Power) | RCRC516012<CR> |
|  | 1600013 | Remote Code:AMP INPUT UP | RCRC51600013<CR> |
|  | 1600014 | Remote Code:AMP INPUT DOWN | RCRC51600014<CR> |
|  | 16016 | Remote Code:AMP VOLUME UP | RCRC516016<CR> |
|  | 16017 | Remote Code:AMP VOLUME DOWN | RCRC516017<CR> |

## ※CD Control

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| BD | CURSOR UP | “Cursor Up” Control | BDCURSOR UP<CR> |
|  | CURSOR DOWN | “Cursor Down” Control | BDCURSOR DOWN<CR> |
|  | CURSOR LEFT | “Cursor Left” Control | BDCURSOR LEFT<CR> |
|  | CURSOR RIGHT | “Cursor Right” Control | BDCURSOR RIGHT<CR> |
|  | ENTER | “Enter” Control | BDENTER<CR> |
|  | PLAY | “Play” Control | BDPLAY<CR> |
|  | PAUSE | “Pause” Control | BDPAUSE<CR> |
|  | PLAY/PAUSE | “Play/Pause” Control | BDPLAY PAUSE<CR> |
|  | STOP | “Stop” Control | BDSTOP<CR> |
|  | SKIP + | “Skip Plus” Control | BDSKIP +<CR> |
|  | SKIP - | “Skip Minus” Control | BDSKIP -<CR> |
|  | MANUAL SEARCH + | “Manual Search Plus” Control (Search Forward) | BDMANUAL SEARCH +<CR> |
|  | MANUAL SEARCH - | “Manual Search Minus” Control (Search Reverse) | BDMANUAL SEARCH -<CR> |
|  | DS TRACK **** | Direct Select(Select track number) ****:0000 to 9999 by ASCII, 0000=number0, 3000= number 3000 ---SYSTEM can be operated from 0 to 9999 | BDDS TRACK 0010<CR> |
|  | OPEN/CLOSE | Disc tray open / closing | BDOPEN/CLOSE<CR> |
|  | REPEAT | “Repeat” setting | BDREPEAT<CR> |
|  | REPEAT ONE | “Repeat One” | BDREPEAT ONE<CR> |
|  | REPEAT ALL | “Repeat All” | BDREPEAT ALL<CR> |
|  | REPEAT OFF | “Repeat Off” | BDREPEAT OFF<CR> |
|  | RANDOM | “RANDOM” Control | BDRANDOM<CR> |
|  | RANDOM ON | “Random On” | BDRANDOM ON<CR> |
|  | RANDOM OFF | “Random Off” | BDRANDOM OFF<CR> |
|  | FOLDER MODE ON | “Folder Mode On” | BDFOLDER MODE ON<CR> |
|  | FOLDER MODE OFF | “Folder Mode Off” | BDFOLDER MODE OFF<CR> |
|  | FOLDER MODE | “Folder Mode On/Off” Toggle | BDFOLDER MODE<CR> |
|  | FOLDER NAME? | “FOLDER NAME”Request | BDFOLDER NAME?<CR> |
|  | FILE NAME? | “FILE NAME”Request | BDFILE NAME?<CR> |
|  | ARTIST NAME? | “ARTIST NAME”Request | BDARTIST NAME?<CR> |
|  | ALBUM NAME? | “ALBUM NAME”Request | BDALBUM NAME?<CR> |
|  | SONG NAME? | “SONG NAME”Request | BDSONG NAME?<CR> |
|  | STATUS? | Acquires system status (such as the entire player and transfer unit). | BDSTATUS?<CR> |
|  | FOLDER + | “Folder Plus” Control | BDFOLDER +<CR> |
|  | FOLDER - | “Folder Minus” Control | BDFOLDER -<CR> |
|  | 1 | 1 | BDKEY 1<CR> |
|  | 2 | 2 | BDKEY 2<CR> |
|  | 3 | 3 | BDKEY 3<CR> |
|  | 4 | 4 | BDKEY 4<CR> |
|  | 5 | 5 | BDKEY 5<CR> |
|  | 6 | 6 | BDKEY 6<CR> |
|  | 7 | 7 | BDKEY 7<CR> |
|  | 8 | 8 | BDKEY 8<CR> |
|  | 9 | 9 | BDKEY 9<CR> |
|  | 0 | 0 | BDKEY 0<CR> |
|  | +10 | +10 | BDKEY 10<CR> |
|  | CLEAR | CLEAR | BDCLEAR<CR> |

# EVENT(or RESPONSE) and PARAMETER list

## ※SYSTEM関係

| EVENT | PARAMETER | Function | example |
| --- | --- | --- | --- |
| PW | ON | POWER ON/STANDBY change | PWON<CR> |
|  | STANDBY |  | PWSTANDBY<CR> |
| MV | ** | MASTER VOLUME change , | MV80<CR> |
|  |  | **:00 to 60 by ASCII |  |
| MU | ON | OUTPUT MUTE ON/OFF change | MUON<CR> |
|  | OFF |  | MUOFF<CR> |
| SI | NET | Heos Favorites | SINET<CR> |
|  |  | Internet Radio |  |
|  |  | Music Server |  |
|  |  | USB |  |
|  |  | Music Services |  |
|  | BT | Bluetooth | SIBT<CR> |
|  | CD | CD | SICD<CR> |
|  | PC | PC(USB-DAC) | SIPC<CR> |
|  | COAXIAL | Coaxial | SICOAXIAL<CR> |
|  | OPTICAL1 | Optical 1 | SIOPTICAL1<CR> |
|  | OPTICAL2 | Optical 2 | SIOPTICAL2<CR> |
| SLP | OFF | MAIN ZONE SLEEP TIMER setting change | SLPOFF<CR> |
|  | *** |  | SLP090<CR> |

## ※FAVORITE Control : CLI use (This part cannot operate via Serial RS232C control. Same as AVR.)

## ※Variable Output

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | VVL | Variable Output（Volume Limit）:0dB | SSVVL 000 |
|  |  | Variable Output（Volume Limit）:-10dB | SSVVL 010 |
|  |  | Variable Output（Volume Limit）:-20dB | SSVVL 020 |
|  |  | Volume Limit Direct | SSVVL *** （***：Limit） |

## ※Auto Standby

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | STB | Auto Standby：ON | SSSTB ON |
|  |  | Auto Standby：OFF | SSSTB OFF |
|  |  | Auto Standby：1min | SSSTB 01MIN |
|  |  | Auto Standby：5min | SSSTB 05MIN |
|  |  | Auto Standby：15min | SSSTB 15MIN |
|  |  | Auto Standby Direct | SSSTB *** （***：TIME） |

## ※Variable Output Volume

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| MV | UP | MASTER VOLUME UP/DOWN , direct change to **dB | MVUP<CR> |
|  | DOWN |  | MVDOWN<CR> |
|  | ** or *** | 00 to 100 by ASCII (0.5step) | MV100<CR> 100.0 MV995<CR> 99.5 MV99<CR> 99.0 MV985<CR> 98.5 MV98<CR> 98.0 : : : : MV015<CR> 1.5 MV01<CR> 1.0 MV005<CR> 0.5 MV00<CR> 0.0 |

## ※Language設定

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | LAN ENG | 英語 | SSLAN ENG<CR> |
|  | LAN DEU | ドイツ語 | SSLAN DEU<CR> |
|  | LAN FRA | フランス語 | SSLAN FRA<CR> |
|  | LAN ITA | イタリア語 | SSLAN ITA<CR> |
|  | LAN ESP | スペイン語 | SSLAN ESP<CR> |
|  | LAN NER | オランダ語 | SSLAN NER<CR> |
|  | LAN SVE | スウェーデン語 | SSLAN SVE<CR> |
|  | LAN JPN | 日本語 | SSLAN JPN<CR> |
|  | LAN CHI | 中国語 | SSLAN CHI<CR> |
|  | LAN POL | ポーランド語 | SSLAN POL<CR> |
|  | LAN RUS | ロシア語 | SSLAN RUS<CR> |

## ※AUDIO 再生フォーマット

| COMMAND | PARAMETER1 | PARAMETER2 | Function | example |
| --- | --- | --- | --- | --- |
| SS | FMTLPC | 032 | 再生ファイルの種類(LPCM:32kHz) | SSFMTLPC032<CR> |
|  |  | 044 | 再生ファイルの種類(LPCM:44.1kHz) | SSFMTLPC044<CR> |
|  |  | 048 | 再生ファイルの種類(LPCM:48kHz) | SSFMTLPC048<CR> |
|  |  | 088 | 再生ファイルの種類(LPCM:88.2kHz) | SSFMTLPC088<CR> |
|  |  | 096 | 再生ファイルの種類(LPCM:96kHz) | SSFMTLPC096<CR> |
|  |  | 176 | 再生ファイルの種類(LPCM:176.4kHz) | SSFMTLPC176<CR> |
|  |  | 192 | 再生ファイルの種類(LPCM:192kHz) | SSFMTLPC192<CR> |
|  |  | 352 | 再生ファイルの種類(LPCM:352.8kHz) | SSFMTLPC352<CR> |
|  |  | 384 | 再生ファイルの種類(LPCM:384kHz) | SSFMTLPC384<CR> |
|  | FMTMP3 | 032 | 再生ファイルの種類(MP3:32kHz) | SSFMTMP3032<CR> |
|  |  | 044 | 再生ファイルの種類(MP3:44.1kHz) | SSFMTMP3044<CR> |
|  |  | 048 | 再生ファイルの種類(MP3:48kHz) | SSFMTMP3048<CR> |
|  | FMTWMA | 032 | 再生ファイルの種類(WMA:32kHz) | SSFMTWMA032<CR> |
|  |  | 044 | 再生ファイルの種類(WMA:44.1kHz) | SSFMTWMA044<CR> |
|  |  | 048 | 再生ファイルの種類(WMA:48kHz) | SSFMTWMA048<CR> |
|  | FMTAAC | 032 | 再生ファイルの種類(AAC:32kHz) | SSFMTAAC032<CR> |
|  |  | 044 | 再生ファイルの種類(AAC:44.1kHz) | SSFMTAAC044<CR> |
|  |  | 048 | 再生ファイルの種類(AAC:48kHz) | SSFMTAAC048<CR> |
|  | FMTFLC | 032 | 再生ファイルの種類(FLAC:32kHz) | SSFMTFLC032<CR> |
|  |  | 044 | 再生ファイルの種類(FLAC:44.1kHz) | SSFMTFLC044<CR> |
|  |  | 048 | 再生ファイルの種類(FLAC:48kHz) | SSFMTFLC048<CR> |
|  |  | 088 | 再生ファイルの種類(FLAC:88.2kHz) | SSFMTFLC088<CR> |
|  |  | 096 | 再生ファイルの種類(FLAC:96kHz) | SSFMTFLC096<CR> |
|  |  | 176 | 再生ファイルの種類(FLAC:176.4kHz) | SSFMTFLC176<CR> |
|  |  | 192 | 再生ファイルの種類(FLAC:192kHz) | SSFMTFLC192<CR> |
|  | FMTALC | 032 | 再生ファイルの種類(ALAC:32kHz) | SSFMTALC032<CR> |
|  |  | 044 | 再生ファイルの種類(ALAC:44.1kHz) | SSFMTALC044<CR> |
|  |  | 048 | 再生ファイルの種類(ALAC:48kHz) | SSFMTALC048<CR> |
|  |  | 088 | 再生ファイルの種類(ALAC:88.2kHz) | SSFMTALC088<CR> |
|  |  | 096 | 再生ファイルの種類(ALAC:96kHz) | SSFMTALC096<CR> |
|  |  | 176 | 再生ファイルの種類(ALAC:176.4kHz) | SSFMTALC176<CR> |
|  |  | 192 | 再生ファイルの種類(ALAC:192kHz) | SSFMTALC192<CR> |
|  | FMTAIF | 032 | 再生ファイルの種類(AIFF:32kHz) | SSFMTAIF032<CR> |
|  |  | 044 | 再生ファイルの種類(AIFF:44.1kHz) | SSFMTAIF044<CR> |
|  |  | 048 | 再生ファイルの種類(AIFF:48kHz) | SSFMTAIF048<CR> |
|  |  | 088 | 再生ファイルの種類(AIFF:88.2kHz) | SSFMTAIF088<CR> |
|  |  | 096 | 再生ファイルの種類(AIFF:96kHz) | SSFMTAIF096<CR> |
|  |  | 176 | 再生ファイルの種類(AIFF:176.4kHz) | SSFMTAIF176<CR> |
|  |  | 192 | 再生ファイルの種類(AIFF:192kHz) | SSFMTAIF192<CR> |
|  | FMTDSD | 028 | 再生ファイルの種類(DSD:2.8MHz) | SSFMTDSD028<CR> |
|  |  | 030 | 再生ファイルの種類(DSD:3.0MHz) | SSFMTDSD030<CR> |
|  |  | 056 | 再生ファイルの種類(DSD:5.6MHz) | SSFMTDSD056<CR> |
|  |  | 061 | 再生ファイルの種類(DSD:6.1MHz) | SSFMTDSD061<CR> |
|  |  | 112 | 再生ファイルの種類(DSD:11.2MHz) | SSFMTDSD112<CR> |
|  |  | 122 | 再生ファイルの種類(DSD:12.2MHz) | SSFMTDSD122<CR> |
|  | FMTULC |  | Signal Unlock | SSFMTULC<CR> |
|  | FMTUSP |  | Unsupported | SSFMTUSP<CR> |

## ※DIMMER設定

| COMMAND | PARAMETER | Function | example |
| --- | --- | --- | --- |
| SS | DIM000 | DIMMER：0％ | SSDIM000<CR> |
|  | DIM025 | DIMMER：25％（5段階用） | SSDIM025<CR> |
|  | DIM040 | DIMMER：40％（4段階用） | SSDIM040<CR> |
|  | DIM050 | DIMMER：50％（5段階用） | SSDIM050<CR> |
|  | DIM070 | DIMMER：70％（4段階用） | SSDIM070<CR> |
|  | DIM075 | DIMMER：75％（5段階用） | SSDIM075<CR> |
|  | DIM100 | DIMMER：100％ | SSDIM100<CR> |

## ※Network Related : CLI use (This part cannot operate via Serial RS232C control. Same as AVR.)

## ※CD Control

| EVENT | PARAMETER | function | example |
| --- | --- | --- | --- |
| BD | STATUS *+-#¦/abcdddeeeefhhmmssM | Acquires system status (such as the entire player and transfer unit). (*A) | BDSTATUS ”SP”123456731002000811223300<CR> |
|  |  | *:”SP”(20h),0 to 3 by ASCII(Answer code) ( *1 ) |  |
|  |  | +:1 to ? by ASCII(Disc type code) ( *2 ) |  |
|  |  | -:(Audio format code) ( *3 ) |  |
|  |  | #:(Audio channel code) ( 2ch：32ｈ（2）固定 ) |  |
|  |  | ¦:”0” |  |
|  |  | /:”0” |  |
|  |  | a:”0” |  |
|  |  | b:(Status code) ( *6 ) |  |
|  |  | c:(Play mode code) ( *5) |  |
|  |  | ddd:”0” |  |
|  |  | eeee:(Track information) |  |
|  |  | f:(Time mode) ( *4) |  |
|  |  | hh:(Elapsed time hour) |  |
|  |  | mm:(Elapsed time minutes) |  |
|  |  | ss:(Elapsed time second) |  |
|  |  | M:(Disc/USB mode) ( *7) |  |
|  | PLAY * | “Play” Control *:”SP”, 0 to 3 by ASCII(Answer code) ( *1 ) | BDPLAY ”SP”<CR> |
|  | PAUSE * | “Pause” Control(*B) *:”SP”,0 to 3 by ASCII (Answer code) ( *1 ) | BDPAUSE 1<CR> |
|  | STOP * | “Stop” Control *:”SP”,0 to 3 by ASCII (Answer code) ( *1 ) | BDSTOP 2<CR> |
|  | SKIP *---- | “Skip” Control (*C) *:”SP”,0 to 3 by ASCII (Answer code) ( *1 ) | BDSKIP ”SP”0000001<CR> |
|  |  | +++:0 |  |
|  |  | ----:0 to 9999 by ASCII(track number) |  |
|  | MANUAL SEARCH * | Manual Search change (*D) | BDMANUAL SEARCH ”SP”<CR> |
|  |  | *:”SP”,0 to 3 by ASCII (Answer code) ( *1 ) |  |
|  | DS TRACK *+++---- | Direct Select | BDDS TRACK ”SP”0000020<CR> |
|  |  | *:”SP”,0 to 3 by ASCII (Answer code) ( *1 ) |  |
|  |  | +++:000 |  |
|  |  | ----:0000 to 9999 by ASCII (track number) |  |
|  | CURSOR * | “Cursor” Control *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDCURSOR ”SP”<CR> |
|  | ENTER | “Enter” Control *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDENTER ”SP”<CR> |
|  | OPEN/CLOSE * | Disc tray open / closing *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDOPEN/CLOSE ”SP”<CR> |
|  | REPEAT * | “Repeat” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDREPEAT ”SP”<CR> |
|  | REPEAT ONE * | “Repeat One” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDREPEAT ONE ”SP”<CR> |
|  | REPEAT ALL * | “Repeat All” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDREPEAT ALL ”SP”<CR> |
|  | REPEAT OFF * | “Repeat Off” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDREPEAT OFF ”SP”<CR> |
|  | RANDOM * | “RANDOM” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDRANDOM ”SP”<CR> |
|  | RANDOM ON * | “RANDOM ON” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDRANDOM ON ”SP”<CR> |
|  | RANDOM OFF * | “RANDOM OFF” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDRANDOM OFF ”SP”<CR> |
|  | FOLDER MODE ON * | “FOLDER MODE ON” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDFOLDER MODE ON ”SP”<CR> |
|  | FOLDER MODE OFF * | “FOLDER MODE OFF” Setting *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) | BDFOLDER MODE OFF ”SP”<CR> |
|  | FOLDER NAME *aaaaa_???? | “FOLDER NAME” Request *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) a:Character Length(MAX32byte) _:Null ?: Don't Care (The character after Null should be disregarded) ************_?????:33byte Fixed | BDFOLDER NAME ”SP” Norah Jones_???????????<CR> |
|  | FILE NAME *aaaaa_???? | “FILE NAME” Request *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) a:Character Length(MAX32byte) _:Null ?: Don't Care (The character after Null should be disregarded) ************_?????:33byte Fixed | BDFILE NAME ”SP” Come Away With Me.mp3_?<CR> |
|  | ARTIST NAME *aaaaa_???? | “ARTIST NAME” Request *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) a:Character Length(MAX32byte) _:Null ?: Don't Care (The character after Null should be disregarded) ************_?????:33byte Fixed | BDARTIST NAME ”SP” Norah Jones_???????????<CR> |
|  | ALBUM NAME *aaaaa_???? | “ALBUM NAME” Request *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) a:Character Length(MAX32byte) _:Null ?: Don't Care (The character after Null should be disregarded) ************_?????:33byte Fixed | BDALBUM NAME ”SP” Come Away With Me_?<CR> |
|  | SONG NAME *aaaaa_???? | “SONG NAME” Request *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) a:Character Length(MAX32byte) _:Null ?: Don't Care (The character after Null should be disregarded) ************_?????:33byte Fixed | BDSONG NAME ”SP” Come Away With Me_?<CR> |
|  | FOLDER + | “Folder Plus” Control *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) a:Character Length(MAX32byte) _:Null ?: Don't Care (The character after Null should be disregarded) ************_?????:33byte Fixed | BDFOLDER NAME *aaaaa_????<CR> |
|  | FOLDER - | “Folder Minus” Control *:”SP”,0 to 3 by ASCII(Answer code) ( *1 ) a:Character Length(MAX32byte) _:Null ?: Don't Care (The character after Null should be disregarded) ************_?????:33byte Fixed | BDFOLDER NAME *aaaaa_????<CR> |
|  | 1 | 1 | BDKEY 1<CR> |
|  | 2 | 2 | BDKEY 2<CR> |
|  | 3 | 3 | BDKEY 3<CR> |
|  | 4 | 4 | BDKEY 4<CR> |
|  | 5 | 5 | BDKEY 5<CR> |
|  | 6 | 6 | BDKEY 6<CR> |
|  | 7 | 7 | BDKEY 7<CR> |
|  | 8 | 8 | BDKEY 8<CR> |
|  | 9 | 9 | BDKEY 9<CR> |
|  | 0 | 0 | BDKEY 0<CR> |
|  | +10 | +10 | BDKEY 10<CR> |
|  | CLEAR | CLEAR | BDCLEAR<CR> |

(*1) answer codes

| No. | Status | Code(ASCII) | Description |
| --- | --- | --- | --- |
| 1 | - Command OK | - 20h(SP) | - Accepts the command. |
| 2 | - Invalid | - 30h(0) | - Invalid command. |
| 3 | - Order Track None | - 32h(2) | - The track , the group ,the title or the chapter you specified does not exist. |

(*2)Disc type code

| Code | Disc Type |
| --- | --- |
| 34h(4) | CD-DA |
| 35h(5) | CD-ROM |
| 36h(6) | UNKNOWN |
| 40h(A) | USB |
| 41h(B) | iPod |

(*3) Audio format code

| Code | Audio Format |
| --- | --- |
| 34h(4) | LPCM |
| 36h(6) | UNKNOWN |
| 3Bh(;) | MP3 |
| 3Dh(=) | WMA |

(*4) Time Mode code

| Code | Time Mode |
| --- | --- |
| 34h(4) | TOTAL REMAIN |
| 39h(9) | TRACK ELAPSED |
| 3Ah(:) | TRACK REMAIN |
| 3Bh(;) | FOLDER NAME |

(*5) Play mode code

| Code | Play Mode |
| --- | --- |
| 31h(1) | NORMAL |
| 32h(2) | PROGRAM |
| 33h(3) | RANDOM |
| 34h(4) | REPEAT ONE |
| 35h(5) | RANDOM + REPEAT ALL |
| 36h(6) | REPEAT ALL |
| 37h(7) | PROGRAM + REPEAT ALL |
| 38h(8) | PROGRAM + RANDOM |
| 39h(9) | PROGRAM + RANDOM + REPEAT ALL |
| 3A(:) | REPEAT FOLDER |
| 3B(;) | NORMAL + FOLDER |

(*6) status  codes

Here is a list of answer  code  types.

3Xh  : Status  of the  entire  system

4Xh  : Status  of each  action  mode

| No. | Status | Code(ASCII) | Description |
| --- | --- | --- | --- |
| 1 | - Stand-by | - 30h(0) | - Stand-by |
| 2 | - Disc Loading | - 31h(1) | - Under disc loading. |
| 3 | - Disc Loading Complete | - 32h(2) | - Disc Loading complete. |
| 4 | - Tray Opening | - 33h(3) | - Disc tray open. |
| 5 | - Tray Closing | - 34h(4) | - Disc tray close. |
| 6 | - No Disc | - 41h(A) | - Disc not present |
| 7 | - Stop | - 42h(B) | - Stop |
| 8 | - Play | - 43h(C) | - Under disc playing. |
| 9 | - Pause | - 44h(D) | - Playback in process. |
| 10 | - Scan Play | - 45h(E) | - Scanning in process. |
| 11 | - Scan Play(forword) | - 46h(F) | - Scanning in process. |
| 12 | - Scan Play(reverse) | - 47h(G) | - Scanning in process. |

(*7)DISC/USB MODE

| No. | Status | Code(ASCII) | Description |
| --- | --- | --- | --- |
| 1 | - Disc | - 30h(0) |  |
| 2 | - USB | - 31h(1) |  |

Special conditions

(*A) BDSTATUS

- When the disc does not set to DVD mechanism and disc loading process does not finish,  group number, title number, track number, and chapter number are set ('0').

- When the disc does not set to DVD mechanism and disc loading process does not finish,  elapsed time information are set ('0').

- When power condition is “STANDBY”, can accept “REQUEST SYSTEM STATUS”, “POWER ON KEY”,  “REQUEST CPU VERSION”, and “REQUEST ERROR STATUS”. Incase of another command, returns  “COMMAND  FORMAT  ERROR ('1')“  in the “ANSWER CODE”

Note : When you need these data, you should send this command.

(*C) BDSKIP

- This command is valid only , when mode status data is Play (E) or Pause (F).

- The unit can skip to a maximum track with Forward (+) and to a minimum track with Reverse  (–).

(*D) BDMANUAL SEARCH

- This command is valid only when the mode status is Play (E) or Pause (F).